## Introduction

With the  PSR-8000 we can save parameters of the keyboard on a disk in order to get back these later if wanted. Using the function ‚SAVE TO DISK‘ there are eight types of data to be stored separately: SETUP, EFFECT DATA, REGISTRATION BANK, GROOVE STYLE, MULTI PAD, CUSTOM STYLE, CUSTOM VOICE and LINK TO SONG.

The aim of this article is a description of the structure of archives  for registration-banks, which are stored an a floppy disk. I have analyzed a little part only. Some information, especially about the codes of styles, I got from Ulrich Montag ( u.montag@t-online.de ) , who gave me the idea to make these researches.

Remark:

The files of archives are written by filenames without the delimiter ‚.‘ at the beginning of the file-extension. All numbers in the textfiles are decimal ones.

In this article I use the denotation of hexadecimal numbers with starting ‚0x‘ or ending ‚h‘.

## The Structure of Registration Archives

We can save several registration archives on only one storage medium, hard  or floppy disk, using a different archive-name for each one. The PSR-8000  takes the default-names UF_00001, UF_00002,..., but the user can choose own names.

If there are stored  n, n = 1,2,..., registration-archives UF_00001, UF_00002,..,UF_0000n on a floppy disk, the PSR_8000 has created the following files in the main-directory:

Disk.mng

On all floppy disks, which have got some data by the PSR-8000, we find this file with the same text inside. The file is used by the keyboard to indentify a PSR-8000 disk. With this examination the software will get some security before reading  the other files.

UF_00001.usr, UF_00002.usr,...UF_0000n.usr

These text-files can be read by an text-editor. Each file delivers some informations for only one archive. We find filenames inside, pointing to other files of the archives of the floppy disk. The files are used with all types of archives, but here we consider registration archives only.The files consist of blocks with the length of 16 bytes, that means that every line starts with a character with the byte-adress n*16. If we see less then 14 characters in a line, the line is filled with blanks. The lines end with the characters CR and LF (0x0d, 0x0a). There is one line with more than one 16-byte-block.

These files *.usr start with a file-header. At the beginning we read the actual filename (without delimiter ‚.‘) and it ends with the string ‚P,I‘. We find the keyboard-modelname PSR-8000 and a version-string. I did not try to get more informations about the header.

After an empty line we find a decimal number of eight digits. This number is a measure of the volume of the archiv. Possibly it is only needed to show the process of loading the archive and it is not needed by the function.

The content of the next line is the word SECTION08. Related with ‚08‘ we know, that there follow eight lines of same structure xxx,nn,mm,pppp . xxx is one of the abbreviations of the type of saved informations: MID, SET, REG, PAD, STY, VIC, TWK and EFF. nn, mm and pppp denote decimal numbers. Because we only have saved registrations, there is nn = 00 and mm = 00 for all xxx not equal REG: because the numbers nn and mm in the line of REG denote the number of saved registration banks. This number is used by the software of the PSR-8000 to identify the information in a later part of the file, which starts with the key REG at byte-number pppp . Here we find nn filenames RPKcccyyREG. ccc is one of the numbers 001, 002, 003,...; with a constant value in a file *.usr. The numbers yy = 01, 02,...,15,16 denote the bank-numbers of the registration saved. If we add n RPK-files to the list REG, we have to add the number n*16 to each pppp of the following lists PAD, STY, VIC, TWK and EFF.

The next line after „EFF,mm,nn,pppp“ looks like

00000000,00000092,00000000,00000000,00000000,10\

for example. We have to regard in pure registration-archives the last four hexa-digits of the second number, here 0092. This number is bitwise denoted as 0000 0000 1001 0010. Each bit set marks an existing RPK-File, to be read from the right. Here there exist the three files RPKxxxyy.REG,  yy = 02, 05, 08.

Example:

In an archive with the user-defined name REG01 we have saved the four registration banks 2,3,5 and 15. On the floppy disk we find the file REG01.usr. Behind the line SECTION08 there is the line REG,04,04,0368 with some digits pppp. In the list, beginning with the line REG we find the four filenames RPK00102REG, RPK00103REG, RPK00105REG and RPK00115REG. xxx = 001 here is valid only, if the archive REG01 is the first one created on the floppy disk.

The File Reg.mng

All files RPKxxxyy.REG of different archives on a floppy disk are stored in only one directory Reg.

In Reg there is an additional file Reg.mng with the following structure:

File-Header:

REG     MNG

FILEiii

FILE00000iii

iii is the number of all RPK-files of the directory Reg.

After this there follows for each RPK-File a block of same structure:

RPKxxxyyREG

PSR-8000

VER0.01Rev0.01

A,I

00002689

There last decimal number may be constant in all registration archives of the PSR-8000.

The Structure Of The Files RPKxxxyy.REG

These files have the fixed number of 0x0A81 binary data-bytes: a header, eight data-blocks and a trailer, each with a fixed length and a welldefined structure.

The data is stored at the following byte-numbers:

| Block | Byte-Number | Length |
| --- | --- | --- |
| Header | 0x0000 | 0x0080 |
| Data block 1 | 0x0080 | 0x013C |
| Data block 2 | 0x01BC | 0x013C |
| Data block 3 | 0x02F8 | 0x013C |
| Data block 4 | 0x0434 | 0x013C |
| Data block 5 | 0x0570 | 0x013C |
| Data block 6 | 0x06AC | 0x013C |
| Data block 7 | 0x07E8 | 0x013C |
| Data block 8 | 0x0924 | 0x013C |
| Trailer | 0x0A60 | 0x0021 |
| Summary |  | 0x0A81 |

The File-Header of  RPKxxxyy.REG

The first block is a header with a length of 0x80 bytes. The content of the header are the same ASCII-characters as in the file reg.mng:

Example (Registration-Bank 13):

00000000 5250 4B30 3032 3133 5245 4720 2020 0D0A RPK00213REG   ..

00000010 2020 2020 2020 2020 2020 2020 2020 2020

00000020 2020 2020 2020 2020 2020 2020 2020 0D0A               ..

00000030 5053 522D 3830 3030 2020 2020 2020 0D0A PSR-8000      ..

00000040 5665 7230 2E30 3152 6576 302E 3031 0D0A Ver0.01Rev0.01..

00000050 412C 4920 2020 2020 2020 2020 2020 0D0A A,I           ..

00000060 2020 2020 2020 2020 2020 2020 2020 0D0A               ..

00000070 2020 2020 2020 2020 2020 2020 2020 0D0A               ..

The Data-Blocks of  RPKxxxyy.REG

The next data following the header are eight blocks, each of these with a length of 0x13C bytes. The block with the number ‚n‘ consists all saved informations of the part ‚n‘ of the registration bank.

Example of a data-block:

00000000 0022 0084 00DC 007D 0059 0205 0611 7FC0 .".....}.Y......

00000010 A0A0 A001 0020 005A 007F 1E36 0010 0C2F ..... .Z...6.../

00000020 0C00 0101 0007 0707 0000 0000 0007 0100 ................

00000030 0000 0000 3636 0200 FF00 1F00 2464 050E ....66......$d..

00000040 0017 64FF 02FF 4601 FF03 7F00 021C 5E7F ..d...F.......^.

00000050 7F01 0000 0000 0101 0101 0101 0101 4040 ..............@@

00000060 4040 4040 4040 2D40 4040 1C10 1C16 5A46 @@@@@@-@@@....ZF

00000070 4646 1C28 230D 0202 4040 4040 1F0F F0F0 FF.(#...@@@@....

00000080 0202 0202 4040 4040 4040 4040 0028 1E24 ....@@@@@@@@.(.$

00000090 0006 000B 000B 0000 0098 FFFF 372D 3220 ............7-2

000000A0 4327 6573 7420 6369 2062 6F6E 4E6F 7420 C'est ci bonNot

000000B0 5365 6C65 6374 6564 4040 0040 4040 4040 Selected@@.@@@@@

000000C0 4040 2340 4040 4040 9207 0A7F D800 0001 @@#@@@@@........

000000D0 0100 0000 0000 0000 0000 0000 0000 2936 ..............)6

000000E0 0063 6357 6162 2946 5E40 4040 4040 4040 .ccWab)F^@@@@@@@

000000F0 4040 4001 0040 4040 1440 4064 2340 4040 @@@..@@@.@@d#@@@

00000100 4040 4040 4040 4040 4040 4040 4040 4040 @@@@@@@@@@@@@@@@

00000110 3C40 4040 5A2A 4040 2323 0035 1950 0000 <@@@Z*@@##.5.P..

00000120 0000 001E 0000 0000 0000 0000 0000 2D3A ..............-:

00000130 0101 1E03 0540 4040 0001 7F03           .....@@@....

The Trailer of  RPKxxxyy.REG

The file ends with a trailer of 0x0021 bytes at byte-number 0x0A60.

00000000 FF00 9055 0000 9054 4F81 7090 5400 0090 ...U...TO.p.T...

00000010 534F 8170 9053 0000 9054 4F00 923C 4F00 SO.p.S...TO..<O.

00000020 92                                      .

Maybe the Trailer has a fixed content.

The Structure of RPKxxxyy.REG-Data-Blocks

We read in the manual (Parameter List, Row Registration) about all  information to be stored in the data-blocks, but we find neither the code nor the byte-position of each data there. In the next list there is only a little part of these informations.

| Byte-Adresse | Byteanzahl | Inhalt | Beschreibung |
| --- | --- | --- | --- |
| 0x0000 | 0x0002 | Voice# Left | Voice# |
| 0x0002 | 0x0002 | Voice# Right 1 | Voice# |
| 0x0004 | 0x0002 | Voice# Right 2 | Voice# |
| 0x0006 | 0x0002 | Voice# Lead | Voice# |
| 0x0010 | 0x0001 | Voice Effect 1 | Voice Effect 1 |
| 0x000D | 0x0001 | Part Select/ON-OFF | Part Select |
| 0x009C | 0x0010 | Name of the registration | Name of the registartion |
| 0x00AC | 0x000C | Name of selected song | Name of selected song |
| 0x00C8 | 0x0001 | Stylecode | Stylecode |
| 0x00C9 | 0x0001 | Category | Category |
| 0x00CA | 0x0001 | Preset1#-1 | Preset1# |

Voice#:

Voice Number, (Preset 1 Order) -1 (Manual)

Back to the data-block structure

Voice Effect 1:

The byte Voice Effect 1 has an analytic code.

The first nibble is the LED Voice Effect 1, the second has the value 0x0 perhaps.

| Nibble 1 | LED ON for |
| --- | --- |
| 0x8 | REVERB(1) |
| 0x4 | CHORUS(2) |
| 0x2 | DSP(4-6) |
| 0x1 | DSP VARIATION |

Example:

The byte ‚Voice Effect 1‘ (byte-number 0x0010) is 0xA0. Because 0xA0 = 0x80 + 0x20, the states of Reverb(1) and DSP(4-6) are ON.

Back to the data-block structure

Part Select:

The byte Part Select/ON-OFF has an analytic code.

The first nibble is the state of LED Part Select.

| Nibble 1 | LED ON for |
| --- | --- |
| 0x8 | Left |
| 0x4 | Right 1 |
| 0x2 | Right 2 |
| 0x1 | Lead |

The second nibble is the state of ON-Parts.

| Nibble 2 | Part ON |
| --- | --- |
| 0x8 | Left |
| 0x4 | Right 1 |
| 0x2 | Right 2 |
| 0x1 | Lead |

Example:

The byte ‘Part Select‘ (byte-number 0x000D) has the value 0x46. With 4 there is Right 1 selected. With the second nibble  0x6 the bits 0x4 and 0x2 are set. Therefore the states of Right 1 and Right 2 are ON,  Left and Lead are OFF.

Back to the data-block structure

Name Of  Registration:

Name of the registration (ASCII ).

Back to the data-block structure

Name Of Selected Song:

Name of selected song (ASCII). The PSR-8000 has the function to save selected songs of the song-list in the registrations. After switching on such a registration, the play of the selected midifile starts immediately with the START-Knob.

Remarks: With the name of the song, the directory and the song-number in the directory is stored, but I did not analyze this.

Back to the data-block structure

Stylecode:

| Style | Code |
| --- | --- |
| 4/4 Blues | 00h |
| Country Rock 4 | 01h |
| Guitar Ballad | 02h |
| 16 Beat 1 | 03h |
| 8 Beat 4 | 04h |
| 8 Beat Adria | 05h |
| 8 Beat 3 | 06h |
| 8 Beat 1 | 07h |
| Heart Beat | 08h |
| Sunny Rock | 09h |
| Rock 4 | 0Ah |
| Cowboy Rock | 0Bh |
| Rock 3 | 0Ch |
| 8 Beat Soul | 0Dh |
| 8 Beat Heat | 0Eh |
| Folk Rock | 0Fh |
| Hard Rock | 10h |
| Pop Rock 1 | 11h |
| Pop Rock 2 | 12h |
| Pop Rhumba | 13h |
| R & B | 14h |
| Rock Classic | 15h |
| Soul | 16h |
| Twist 2 | 17h |
| 8 Beat Soft | 18h |
| 8 Beat 2 | 19h |
| Rock 2 | 1Ah |
| Soft Rock | 1Bh |
| Epic Ballad | 1Ch |
| Organ Ballad | 1Dh |
| Love Song | 1Eh |
| Blues Rock | 1Fh |
| Rock Shuffle 2 | 20h |
| Boogie 2 | 21h |
| Pop Shuffle1 | 22h |
| Rock & Roll | 23h |
| Blues Shuffle | 24h |
| Shuffle | 25h |
| 16 Beat 2 | 26h |
| 8 Beat Pop | 27h |
| West End | 28h |
| Hip Hop Pop | 29h |
| Dance Pop 3 | 2Ah |
| Funky Pop | 2Bh |
| Rock Ballad 2 | 2Ch |
| Jazz Rock | 2Dh |
| LA Shuffle | 2Eh |
| Fusion 2 | 2Fh |
| 16 Beat Ballad 2 | 30h |
| 16 Beat Ballad 1 | 31h |
| Pop Ballad 1 | 32h |
| 16 Beat 3 | 33h |
| Piano Ballad | 34h |
| Pop Ballad 2 | 35h |
| Pop Ballad 3 | 36h |
| Analog Pop | 37h |
| Pop Ballad 4 | 38h |
| 16 Beat 4 | 39h |
| Rock Ballad 3 | 3Ah |
| Rock Ballad 1 | 3Bh |
| Slow Ballad | 3Ch |
| 80's Rock | 3Dh |
| Cool Night | 3Eh |
| Soul Shuffle | 3Fh |
| Game Show | 40h |
| Funk | 41h |
| Funky Fusion | 42h |
| Fusion 1 | 43h |
| Soft Fusion | 44h |
| Street Pop | 45h |
| 6/8 Ballad | 46h |
| 6/8 Blues 1 | 47h |
| 6/8 Rock | 48h |
| Modern 6/8 | 49h |
| Pop Shuffle2 | 4Ah |
| Slow Rock 1 | 4Bh |
| 6/8 Blues 2 | 4Ch |
| Slow Rock 2 | 4Dh |
| Slow Rock 3 | 4Eh |
| Blues Ballad | 4Fh |
| Cool Dance | 50h |
| Clubdance 2 | 51h |
| Clubdance 1 | 52h |
| Funky Dance | 53h |
| Dance Pop 1 | 54h |
| Entrance | 55h |
| Electro Pop | 56h |
| Disco Clap | 57h |
| Dance Soul | 58h |
| Disco Latin | 59h |
| Euro House | 5Ah |
| Disco Pop | 5Bh |
| Eurobeat | 5Ch |
| Groundbeat | 5Dh |
| Hip Hop | 5Eh |
| Techno 1 | 5Fh |
| Synth Boogie | 60h |
| Techno 2 | 61h |
| Techno 3 | 62h |
| Trance 1 | 63h |
| Trance 2 | 64h |
| Trip Hop | 65h |
| 70's Disco 3 | 66h |
| 70's Disco 2 | 67h |
| 70's Disco 1 | 68h |
| Disco Slap | 69h |
| Party Pop | 6Ah |
| Disco Girls | 6Bh |
| Disco Fox | 6Ch |
| Disco Hands | 6Dh |
| Disco Queen | 6Eh |
| Disco Rock | 6Fh |
| Disco Samba | 70h |
| Disco Philly | 71h |
| Disco Tropic | 72h |
| Meneito | 73h |
| Polka Pop 3 | 74h |
| Polka Pop 1 | 75h |
| Polka Pop 2 | 76h |
| Country Rock 1 | 77h |
| Bluegrass 2 | 78h |
| Bluegrass 1 | 79h |
| Country 2/4 | 7Ah |
| Two Step | 7Bh |
| Country Rock 3 | 7Ch |
| Country Ballad | 7Dh |
| Country Rock 2 | 7Eh |
| Country Shuffle 1 | 7Fh |
| Country Shuffle 2 | 80h |
| Country Swing 2 | 81h |
| Cowboy Boogie | 82h |
| Light Pop | 83h |
| Bebop | 84h |
| Big Band Jazz | 85h |
| Big Band Swing 3 | 86h |
| Big Band Swing 1 | 87h |
| Big Band Swing 2 | 88h |
| Boogie 1 | 89h |
| Cool Jazz | 8Ah |
| Dixieland 1 | 8Bh |
| Dixieland 2 | 8Ch |
| Gypsy Swing | 8Dh |
| Jazz Quartet | 8Eh |
| Lazy Jive | 8Fh |
| Ragtime | 90h |
| Swing 1 | 91h |
| Swing 2 | 92h |
| Swing Ballad | 93h |
| Beguine 1 | 94h |
| More Bossa | 95h |
| Bossa Nova 1 | 96h |
| Espagnol | 97h |
| Jazz Samba | 98h |
| Lambada | 99h |
| Pop Cha Cha | 9Ah |
| Mambo 2 | 9Bh |
| Mambo 1 | 9Ch |
| Merengue | 9Dh |
| Pop Bossa 1 | 9Eh |
| Pop Bossa 2 | 9Fh |
| Pop Reggae | A0h |
| Reggae 2 | A1h |
| Salsa | A2h |
| Samba Rio | A3h |
| Swing Reggae | A4h |
| Big Band Quickstep | A5h |
| Cha Cha Cha | A6h |
| Quickstep | A7h |
| Foxtrot 2 | A8h |
| Foxtrot 3 | A9h |
| Foxtrot 1 | AAh |
| Foxtrot 4 | ABh |
| Pasodoble | ACh |
| Spanish Pasodoble | ADh |
| Rhumba 1 | AEh |
| Rhumba 2 | AFh |
| Samba | B0h |
| Big Band Boogie | B1h |
| Slowfox | B2h |
| Tango Argentina | B3h |
| Tango | B4h |
| March 1 | B5h |
| March 2 | B6h |
| Musette | B7h |
| Overture | B8h |
| Polka 1 | B9h |
| Polka 2 | BAh |
| Oberkrainer Polka | BBh |
| Tarantella | BCh |
| Tejano | BDh |
| Big Band Waltz | BEh |
| Country Waltz | BFh |
| English Waltz 1 | C0h |
| Traditional Waltz 2 | C1h |
| Jazz Waltz | C2h |
| English Waltz 2 | C3h |
| Pop Waltz | C4h |
| Swing Waltz 2 | C5h |
| Swing Waltz 1 | C6h |
| Traditional Waltz 1 | C7h |
| Vienna Waltz | C8h |
| Oberkrainer Walzer | C9h |
| Dance Pop 2 | CAh |
| Rave | CBh |
| Soul Dance | CCh |
| Baroque | CDh |
| Reggae 1 | CEh |
| Country Swing 1 | CFh |
| Twist 1 | D0h |
| Jive | D1h |
| Rock Shuffle 1 | D2h |
| Rock 1 | D3h |
| Bossa Nova 2 | D4h |
| 6/8 March | D5h |
| Custom 1 | D6h |

I have not tried to find out the codes of the other Custom Styles and the Groove Styles.

Back to the data-block structure

Category:

| Number of Style Group: | Style Group |
| --- | --- |
| 0x00 | 8BEAT1 |
| 0x01 | 8BEAT2 |
| 0x02 | 16BEAT |
| 0x03 | BALLAD |
| 0x04 | ROCK |
| 0x05 | DANCEFLOOR |
| 0x06 | DISCO |
| 0x07 | SWING&JAZZ |
| 0x08 | R&B |
| 0x09 | COUNTRY |
| 0x0A | LATIN |
| 0x0B | BALLROOM |
| 0x0C | MARCH&WALTZ |
| 0x0D | GROOVE STYLE |
| 0x0F | CUSTOM STYLE |

Back to the data-block structure

Preset1#:

Number of Styles in a Style Group, (Preset 1 Order) -1 (Manual)

Back to the data-block structure

## Final Remarks

I have stopped to analyze the other informations inside of registration archives here, because it is a hard job to find out the complete structures. I do not see any nice application to be programmed using the missed informations. But it may be possible with the informations here to create new archives using an hexa editor or a new software to be programmed. We should do it carefully, because there are some numbers stored in the mng- and usr-files and we do not know the meaning.

End